# Setting up DDC2Bi/ISP Connection for Malibu

The following is a list of procedure that guides the user how to setup DDC2Bi/ serial ISP connection for Malibu:

- Configure the hardware setting:

For DDC2Bi connection, a Genesis DDC2Bi ISP module is required for loading the hex file into the flash.

For Serial ISP connection, there is no special hardware configuration. All it needs is the GProbe hardware connection.

- Open GProbe program. The version of GProbe software should be 4.4.0.1 or above.

- Press “F10” key on the keyboard. A “GProbe Connection Setup” window should pop-up automatically. The configuration for DDC2Bi and Serial ISP are described as follows:

|  | DDC2Bi Connection | Serial Connection |
| --- | --- | --- |
| Protocol | DDC2Bi3b | Serial1 |
| Port | LPT1 (0x378) | COM1 |
| Speed | 140000 | 115200 |

Press “OK” after configuration setting is done.

- On the command window (at the bottom of the GProbe screen), execute the batch file called “FlashRom_8.txt”. The location of this file is under \gm1601fw\ISP \batch\ directory; therefore, the user can type the following line in the command window:

“batch c:\gm1601fw\ISP\batch\ FlashRom_8.txt”

The content of FlashRom_8 text file is described as follows:

debugon

SetBuffer 0x1000 4096

Reset 0

RAMWrite C:\gm1601FW\Isp\batch\isp8.hex

Run 0x580

FlashErase

FastFlashWrite C:\gm1601FW\rd_monitor\object\rd_monitor.hex

Before running the batch file, please make sure that *.hex file exists and being placed under proper directory.  Also other driver files exist in the directory described above.

- Press “board reset” after the downloading process is success. Now you can start up the firmware.

Note

In case of flash procedure failing than there should be followed the procedure:

- User should type the following line in the command window

“Forcesa 5”

- User should reset the board

- User should type the following line in the command window

“batch c:\gm1601fw\ISP\batch\ FlashRom_8.txt”